# Hola Intercom Software Guide

# Revision History

- 09/01/2017: 1st draft

- 09/18/2017: Added buddy_id to cloud api

- 10/02/2017: Added echo request and reply messages

- 10/16/2017: Added 3rd party components and updated messages

- 10/24/2017: Added manuf_config.py. Updated message handler, messages and cloud_api

- 10/25/2017: Added pin assignments. Added MAX17043 Particle Library

- 10/30/2017: Added restarted parameter to i_am message

- 10/31/2017: Added battery_lvl to cloud api. Cloud api now ticks.

- 11/21/2017: Added S1509 I/O Expander.

- 11/27/2017: Added multiple buddy support.

- 11/28/2017: Documented and applied coding convention

- 11/28/2017: Described object roles.

- 11/28/2017: Pin Assignment fixes

- 11/28/2017: Plf_TickerBase class introduced

- 11/29/2017: Added Trace&Debug section

- 12/7/2017: Added volume control

- 12/15/2017: Added battery checker

- 1/4/2018: Added ratetune print group. Intercom_Incoming is now a Ticker class.

- 1/9/2018: Added stuff() method to circularBuffer.

- 1/9/2018: Added COMM_START/STOP+ACK messages

- 1/9/2018: Added recordRequest to Intercom_Outgoing. Renamed transfer() method to run().

- 1/9/2018: Added codec patch version

- 1/11/2018: Added FSM diagrams

- 1/15/2018: Replaced BatteryChecker section with LevelChecker section. Added WifiChecker.

- 1/15/2018: Added Table of Contents

- 1/17/2018: Added getBatteryPct API

- 1/19/2018: Modified battery_pct and wifi_pct cloud API

- 1/21/2018: Added Intercom_Buddy LED FSM

- 1/26/2018: Added DataDump module and expanded Registry section

- 1/26/2018: Expanded shell commands section

Table of Contents

1	Revision History	1

3	Version Control	3

4	Third Party Components	3

5	Shell Commands	4

6	Pin Assignments	4

7	Firmware Coding Convention	5

8	Firmware Design	7

8.1	Ownership	7

8.2	Intercom_Root	9

8.3	Intercom_Incoming	11

8.4	Intercom Outgoing	12

8.5	Intercom_Controller	13

8.6	Intercom_Buddy	14

8.7	Volume Control	16

8.8	Level Checkers	17

8.9	Codec	19

8.10	Intercom Message Handler	20

8.11	Messages	21

8.12	Intercom_CloudAPI	23

8.13	Intercom_ButtonsAndLeds	24

8.14	Trace & Debug	24

8.15	Registry	24

8.16	Data Dump	25

9	Server Side	25

10	Manuf_config.py	27

# Version Control

https://github.com/rlysens/particleIntercom/

# Third Party Components

Draw.io:

https://www.draw.io/

VS1063 codec patch 2.01:

http://www.vlsi.fi/en/support/software/vs10xxpatches.html

SparkFun_MAX17043_Particle_Library: https://github.com/sparkfun/SparkFun_MAX17043_Particle_Library.git

SparkFunSX1509 Library:

https://github.com/sparkfun/SparkFun_SX1509_Arduino_Library

LCM: https://lcm-proj.github.io/ (in source tree)

Name-gen (in source tree)

XTEA: mbedtls-2.5.1

XTEA Python: https://pypi.python.org/pypi/xtea/0.4.0 (in source tree)

particle.py (in source tree)

- Pytz.py

- Dateutil

- Requests

- pexpect

# Shell Commands

Enable/Disable certain print groups:

particle call Intercom1 en|dis_prntgrp messages/ratetune/default

List available data dump modules:

particle call Intercom1 list_ddump

Request a data dump for given module:

particle call Intercom ddump <modulename>

Serial port monitor:

particle serial monitor COM3/COM4

Build and Flash:

python build.py --device Intercom1/Intercom2/all [--flash]

Copy server-side code to Amazon EC2 server:

copypython2amazon.bat

Copy JSON files to server-side

copyjson2amazon.bat

Login to Amazon EC2 server:

ssh2amazonec3.bat

Run Manufacturing Configuration script:

python manuf_config.py [-h,--help] [-c,--skip_claim] [-f,--skip_flash] [-i,--image_filename <image_filename>]

Finds and claims connected devices, sets up Wifi, flashes reference fw image and configures name, buddy name and secret key.

# Pin Assignments

Photon pin assignments:

A5 = SI to codec

A4 = SO to codec

A3 = SCK to codec

A2 = xDCS to codec

A1 = xRESET to codec

A0 = DREQ to codec

DAC = xCS to codec

D0 = SDA to MAX17043 fuel gauge IC and to sx1509 i/o expander

D1 = SCL to MAX17043 fuel gauge IC and to sx1509 i/o expander

D2 = xRST to sx1509 i/o expander

SX1509 I/O Expander pin assignments:

0 = Battery check button

1 = Buddy 0 button

2 = Buddy 1 button

3 = Buddy 2 button

4 = Buddy 0 LED

5 = Buddy 1 LED

6 = Buddy 2 LED

7 = Wifi check button

8 = Vol.Dec.

9 = Vol.Inc.

10 = LED Bar 0

11 = LED Bar 1

12 = LED Bar 2

13 = LED Bar 3

14 = LED Bar 4

# Firmware Coding Convention

- CamelCase

- Types start with capital

- Functions and variables start with lowercase, e.g. topIndex

- Typedefs end with _t, e.g. RegisterEntry_t

- Pointers end with p, e.g. registry

- Private members have leading underscore, e.g. _registryp

- #defines are all caps and underscores, e.g. MAX_KEY_VAL

- ‘Namespace’-like prefixes are separated with an underscore, e.g. Intercom_Outgoing, intercom_outgoing

- Filenames are lowercase with underscores

- _s may be used as string postfix

- When interacting with 3rd party code, the 3rd party coding convention may be used.

# Firmware Design

## Ownership

## Intercom_Root

Intercom_Root is the root of the Intercom object tree.

## Intercom_Incoming

Intercom_Incoming is in charge of the incoming datapath logic.

## Intercom Outgoing

Intercom_Outgoing is in charge of the outgoing datapath logic.

## Intercom_Controller

Intercom_Controller handles the control plane logic that is not Buddy specific.

## Intercom_Buddy

Intercom_Buddy handles all buddy related control plane logic and user I/O.

## Volume Control

Manages volume control buttons and led bar.

## Level Checkers

Base class Intercom_LevelCheckerBase manages check button and led bar. Intercom_WifiChecker and Intercom_BatteryChecker derive from this base class and implement respectively Wifi Level checking and Battery Level checking.

## Codec

Voice codec logic with back-end SPI interaction factored out into a separate module.

## Intercom Message Handler

Sends, receives and dispatches Intercom messages.

## Messages

Note that the message channel is not considered reliable. Messages can get lost and the message protocols must be robust against occasional message loss. All messages except i_am are encrypted using XTEA block cipher. The secret key is configured into the device at manufacturing time.

Encryption modes CBC and ECB are supported (selected at compile time).

## Intercom_CloudAPI

Intercom_CloudAPI represents the API toward the cloud/Hola App.

X=0, 1 or 2

## Intercom_ButtonsAndLeds

Intercom_ButtonsAndLeds encapsulates interaction with buttons and leds. Uses the SX1509 I/O extender library.

## Trace & Debug

- Tracing: plf_utils.h has a Trace print API (PLF_PRINT) with printgroups that can be enabled/disabled via the cloudAPI.

- Asserts: plf_assert in plf_utils.h

- Error return codes: Each module has a unique MODULE_ID to be used as a base for error return codes (e.g. return –(MODULE_ID+1)).

- Event Counting: See plf_even_counter.h. Counters can be displayed by datadumping the stats module.

- Data Dumping: See Data Dump section below.

## Registry

The Registry keeps track of a number of Registry elements (strings) identified by a registry key. Registry elements can be set, retrieved and removed (made invalid). Certain registry elements are persistent (i.e stored in non-volatile memory), others are volatile. See list below.

Keys:

REG_KEY_MY_NAME persistent

REG_KEY_BUDDY_X_NAME persistent

REG_KEY_BUDDY_X_ID volatile

REG_KEY_SECRET_KEY persistent

X=0, 1 or 2

## Data Dump

Other objects can register a data dump function with the Plf_DataDump object. A data dump function implementation prints out the object’s state. Plf_DataDump hooks into the CloudAPI so that the user can request a data dump of any registered object.

# Server Side

Message_Handler is the central object. Intercom_name_to_id table and interom_id_to_intercom table are used for Intercom name to id mapping and looking up corresponding Intercom objects.

MSG_TABLE

msg_id -> msg_handler_fun

Intercom_name_to_id_table

Intercom_name -> intercom_id

Intercom_id_to_intercom_table

Intercom_id -> intercom

Message_Handler

Send()

Receive()

socket

Intercom

sendTo()

setBuddy()

getEncoderCryptoCodec()

getDecoderCryptoCodec

Msg_voice_data_handler

Msg_i_am_
handler

Msg_who_is_
handler

Set_buddy_
handler

1..1

n..1

1..1

Msg_echo_request_data_handler

Msg_echo_reply_data_handler

# Manuf_config.py

Credentials.json

Name_key.json

Manuf_config.py

Particle.py

Name_key_gen.py

Namegen.py