# Hola Intercom Software Guide

# Revision History

- 09/01/2017: 1st draft

- 09/18/2017: Added buddy_id to cloud api

- 10/02/2017: Added echo request and reply messages

- 10/16/2017: Added 3rd party components and updated messages

- 10/24/2017: Added manuf_config.py. Updated message handler, messages and cloud_api

- 10/25/2017: Added pin assignments. Added MAX17043 Particle Library

- 10/30/2017: Added restarted parameter to i_am message

- 10/31/2017: Added battery_lvl to cloud api. Cloud api now ticks.

- 11/21/2017: Added S1509 I/O Expander.

- 11/27/2017: Added multiple buddy support.

- 11/28/2017: Documented and applied coding convention

- 11/28/2017: Described object roles.

- 11/28/2017: Pin Assignment fixes

- 11/28/2017: Plf_TickerBase class introduced

- 11/29/2017: Added Trace&Debug section

- 12/7/2017: Added volume control

- 12/15/2017: Added battery checker

- 1/4/2018: Added ratetune print group. Intercom_Incoming is now a Ticker class.

- 1/9/2018: Added stuff() method to circularBuffer.

- 1/9/2018: Added COMM_START/STOP+ACK messages

- 1/9/2018: Added recordRequest to Intercom_Outgoing. Renamed transfer() method to run().

- 1/9/2018: Added codec patch version

- 1/11/2018: Added FSM diagrams

- 1/15/2018: Replaced BatteryChecker section with LevelChecker section. Added WifiChecker.

- 1/15/2018: Added Table of Contents

- 1/17/2018: Added getBatteryPct API

- 1/19/2018: Modified battery_pct and wifi_pct cloud API

- 1/21/2018: Added Intercom_Buddy LED FSM

- 1/26/2018: Added DataDump module and expanded Registry section

- 1/26/2018: Expanded shell commands section

- 1/28/2018: Added comm state FSM to intercom_buddy section

- 1/30/2018: Reversed relation between buddy and outgoing

- 1/30/2018: FSM updated for buddy and outgoing

- 2/1/2018: Added subsection about IDs.

- 2/2/2018: Replaced Echo mechanism with simplex keep-alive mechanism

Table of Contents

1	Revision History	1

3	Version Control	3

4	Third Party Components	3

5	Shell Commands	4

6	Pin Assignments	4

7	Firmware Coding Convention	5

8	Firmware Design	7

8.1	Ownership	7

8.2	Intercom_Root	9

8.3	Intercom_Incoming	11

8.4	Intercom Outgoing	12

8.5	Intercom_Controller	13

8.6	Intercom_Buddy	14

8.7	Volume Control	17

8.8	Level Checkers	18

8.9	Codec	20

8.10	Intercom Message Handler	21

8.11	Messages	22

8.11.1	Intercom IDs, Source and Destination IDs in Messages	24

8.12	Intercom_CloudAPI	24

8.13	Intercom_ButtonsAndLeds	25

8.14	Trace & Debug	25

8.15	Registry	25

8.16	Data Dump	26

9	Server Side	26

10	Manuf_config.py	28

# Version Control

https://github.com/rlysens/particleIntercom/

# Third Party Components

Draw.io:

https://www.draw.io/

VS1063 codec patch 2.01:

http://www.vlsi.fi/en/support/software/vs10xxpatches.html

SparkFun_MAX17043_Particle_Library: https://github.com/sparkfun/SparkFun_MAX17043_Particle_Library.git

SparkFunSX1509 Library:

https://github.com/sparkfun/SparkFun_SX1509_Arduino_Library

LCM: https://lcm-proj.github.io/ (in source tree)

Name-gen (in source tree)

XTEA: mbedtls-2.5.1

XTEA Python: https://pypi.python.org/pypi/xtea/0.4.0 (in source tree)

particle.py (in source tree)

- Pytz.py

- Dateutil

- Requests

- pexpect

# Shell Commands

Enable/Disable certain print groups:

particle call Intercom1 en|dis_prntgrp messages/ratetune/default

List available data dump modules:

particle call Intercom1 list_ddump

Request a data dump for given module:

particle call Intercom ddump <modulename>

Serial port monitor:

particle serial monitor COM3/COM4

Build and Flash:

python build.py --device Intercom1/Intercom2/all [--flash]

Copy server-side code to Amazon EC2 server:

copypython2amazon.bat

Copy JSON files to server-side

copyjson2amazon.bat

Login to Amazon EC2 server:

ssh2amazonec3.bat

Run Manufacturing Configuration script:

python manuf_config.py [-h,--help] [-c,--skip_claim] [-f,--skip_flash] [-i,--image_filename <image_filename>]

Finds and claims connected devices, sets up Wifi, flashes reference fw image and configures name, buddy name and secret key.

# Pin Assignments

Photon pin assignments:

A5 = SI to codec

A4 = SO to codec

A3 = SCK to codec

A2 = xDCS to codec

A1 = xRESET to codec

A0 = DREQ to codec

DAC = xCS to codec

D0 = SDA to MAX17043 fuel gauge IC and to sx1509 i/o expander

D1 = SCL to MAX17043 fuel gauge IC and to sx1509 i/o expander

D2 = xRST to sx1509 i/o expander

SX1509 I/O Expander pin assignments:

0 = Battery check button

1 = Buddy 0 button

2 = Buddy 1 button

3 = Buddy 2 button

4 = Buddy 0 LED

5 = Buddy 1 LED

6 = Buddy 2 LED

7 = Wifi check button

8 = Vol.Dec.

9 = Vol.Inc.

10 = LED Bar 0

11 = LED Bar 1

12 = LED Bar 2

13 = LED Bar 3

14 = LED Bar 4

# Firmware Coding Convention

- CamelCase

- Types start with capital

- Functions and variables start with lowercase, e.g. topIndex

- Typedefs end with _t, e.g. RegisterEntry_t

- Pointers end with p, e.g. registry

- Private members have leading underscore, e.g. _registryp

- #defines are all caps and underscores, e.g. MAX_KEY_VAL

- ‘Namespace’-like prefixes are separated with an underscore, e.g. Intercom_Outgoing, intercom_outgoing

- Filenames are lowercase with underscores

- _s may be used as string postfix

- When interacting with 3rd party code, the 3rd party coding convention may be used.

# Firmware Design

## Ownership

## Intercom_Root

Intercom_Root is the root of the Intercom object tree.

## Intercom_Incoming

Intercom_Incoming is in charge of the incoming datapath logic.

## Intercom Outgoing

Intercom_Outgoing is in charge of the outgoing datapath logic.

## Intercom_Controller

Intercom_Controller handles the control plane logic that is not Buddy specific.

## Intercom_Buddy

Intercom_Buddy handles all buddy related control plane logic and user I/O.

## Volume Control

Manages volume control buttons and led bar.

## Level Checkers

Base class Intercom_LevelCheckerBase manages check button and led bar. Intercom_WifiChecker and Intercom_BatteryChecker derive from this base class and implement respectively Wifi Level checking and Battery Level checking.

## Codec

Voice codec logic with back-end SPI interaction factored out into a separate module.

## Intercom Message Handler

Sends, receives and dispatches Intercom messages.

## Messages

Note that the message channel is not considered reliable. Messages can get lost and the message protocols must be robust against occasional message loss. All messages except i_am are encrypted using XTEA block cipher. The secret key is configured into the device at manufacturing time.

Encryption modes CBC and ECB are supported (selected at compile time).

### Intercom IDs, Source and Destination IDs in Messages

The Intercom Server assigns IDs to intercoms, starting from 1 upwards. The Intercom Server itself has a ID of 0. The value -1/0xffffffff means ID_UNKNOWN. These IDs are used as source_id and destination_id in message communication.

## Intercom_CloudAPI

Intercom_CloudAPI represents the API toward the cloud/Hola App.

X=0, 1 or 2

## Intercom_ButtonsAndLeds

Intercom_ButtonsAndLeds encapsulates interaction with buttons and leds. Uses the SX1509 I/O extender library.

## Trace & Debug

- Tracing: plf_utils.h has a Trace print API (PLF_PRINT) with printgroups that can be enabled/disabled via the cloudAPI.

- Asserts: plf_assert in plf_utils.h

- Error return codes: Each module has a unique MODULE_ID to be used as a base for error return codes (e.g. return –(MODULE_ID+1)).

- Event Counting: See plf_even_counter.h. Counters can be displayed by datadumping the stats module.

- Data Dumping: See Data Dump section below.

## Registry

The Registry keeps track of a number of Registry elements (strings) identified by a registry key. Registry elements can be set, retrieved and removed (made invalid). Certain registry elements are persistent (i.e stored in non-volatile memory), others are volatile. See list below.

Keys:

REG_KEY_MY_NAME persistent

REG_KEY_BUDDY_X_NAME persistent

REG_KEY_BUDDY_X_ID volatile

REG_KEY_SECRET_KEY persistent

X=0, 1 or 2

## Data Dump

Other objects can register a data dump function with the Plf_DataDump object. A data dump function implementation prints out the object’s state. Plf_DataDump hooks into the CloudAPI so that the user can request a data dump of any registered object.

# Server Side

Message_Handler is the central object. Intercom_name_to_id table and interom_id_to_intercom table are used for Intercom name to id mapping and looking up corresponding Intercom objects.

MSG_TABLE

msg_id -> msg_handler_fun

Intercom_name_to_id_table

Intercom_name -> intercom_id

Intercom_id_to_intercom_table

Intercom_id -> intercom

Message_Handler

Send()

Receive()

socket

Intercom

sendTo()

setBuddy()

getEncoderCryptoCodec()

getDecoderCryptoCodec

Msg_voice_data_handler

Msg_i_am_
handler

Msg_who_is_
handler

Set_buddy_
handler

1..1

n..1

1..1

Msg_keep_alive_handler

Msg_comm_start/stop_handler

Msg_comm_start/stop_ack_handler

# Manuf_config.py

Credentials.json

Name_key.json

Manuf_config.py

Particle.py

Name_key_gen.py

Namegen.py